Microprocessors and Microcontrollers

Arduino Project

Ashish Kumar (2019UCO1518)

Sandeep Jain (2019UCO1522)

Mayank Goel (2019UCO1558)

Gaurav Singhal (2019UCO1571)

Suraj Kumar (2019UCO1559)

Please check out YOUTUBE (link) for live demo.

PROBLEM STATEMENT:

Measuring CO2 Concentration in Air using Arduino and MQ-135 Sensor

WHY WE CHOOSE THIS?

  - The earth’s Atmospheric CO2 level is increasing day by day.

  - The global average atmospheric carbon dioxide in 2019 was 409.8 parts per million and in October-2020 it is 411.29.

  - Carbon dioxide is a key greenhouse gas and responsible for about three-quarters of emissions.

  - So CO2 level monitoring has also started to gain importance.

  - So,we felt a concern about insufficient ventilation in my room, and are curious about environment we live in!

ABOUT PROJECT:

  - Our proposal is a CO2 device that logs the data from the CO2 sensor onto an  OLED Display Module for further analysis.

  - We are going to use an MQ-135 sensor with Arduino to measure the CO2 concentration.

  - The measured CO2 concentration values will be displayed on the OLED module

  - We will also compare the Arduino MQ-135 sensor readings with Infrared CO2 sensor readings.

COMPONENTS REQUIRED:

  - Arduino Nano

  - MQ-135 Sensor

  - Jumper Wires

  - 0.96 inch I2C/IIC 128x64 OLED Display Module 4 Pin

  - Breadboard

  - 22KΩ Resistor

Arduino Nano:

### Arduino Nano Pin Configuration:

| ### Pin Category | ### Pin Name | ### Details |
| --- | --- | --- |
| ### Power | ### Vin , 3.3V,5V,GND | Vin: Input voltage to Arduino when using an external power source (6-12V). 5V: Regulated power supply used to power microcontroller and other components on the board. 3.3V: 3.3V supply generated by on-board voltage regulator. Maximum current draw is 50mA. GND: Ground pins. |
| ### Reset | ### Reset | ### Resets the microcontroller |
| ### Analog Pins | ### A0 - A7 | ### Used to meansure analog voltage iin the range of 0-5V |
| ### Input / output pins | ### Digital pins D0 – D13 | ### Can be used as input or output pins . 0V (low) and 5V (high) |
| ### Serial | ### Rx,Tx | ### Used to receive and transmit TTL serial data |
| ### External interrupt | ### 2,3 | ### To trigger an inteerupt |
| ### PWM (pulse width modulation) | ### 3,5,6,9,11 | ### Provide 8 bit PWM output |
| ### Inbuild LED | ### 13 | ### To turn on the inbuild LED |

### Arduino Nano Technical Specifications:

| ### Operating voltage | ### 5V |
| --- | --- |
| ### Input voltage for Vin pin | ### 7 – 12 V |
| ### Analog input pins | ### 6 (A0 – A5) |
| ### Digital i/o pins | ### 14 |
| ### Frequency (clock speed) | ### 16MHz |
| ### SRAM | ### 2Kb |

MQ-135 Sensor:

There are various gas sensors present in the market . Few of them are:

| Sensor Name | Gas to measure |
| --- | --- |
| MQ-2 | Methane, Butane, LPG, Smoke |
| MQ-3 | Alcohol, Ethanol, Smoke |
| MQ-4 | Methane, CNG Gas |
| MQ-5 | Natural gas, LPG |
| MQ-6 | LPG, butane |
| MQ-7 | Carbon Monoxide |
| MQ-8 | Hydrogen Gas |
| MQ-9 | Carbon Monoxide, flammable gasses |
| MQ131 | Ozone |
| MQ135 | Air Quality |
| MQ136 | Hydrogen Sulphide gas |
| MQ137 | Ammonia |
| MQ138 | Benzene, Toluene, Alcohol, Propane, Formaldehyde gas, Hydrogen |
| MQ214 | Methane, Natural Gas |
| MQ216 | Natural gas, Coal Gas |
| MQ303A | Alcohol, Ethanol, smoke |
| MQ306A | LPG, butane |
| MQ307A | Carbon Monoxide |
| MQ309A | Carbon Monoxide, flammable gas |

We will be using  MQ – 135 for CO2 detection

Pin configuration :

| Pin No: | Pin Name: | Description |
| --- | --- | --- |
| For Module |  |  |
| 1 | Vcc | Used to power the sensor, Generally the operating voltage is +5V. |
| 2 | Ground | Used to connect the module to system ground. |
| 3 | Digital Out | You can also use this sensor to get digital output from this pin, by setting a threshold value using the potentiometer. |
| 4 | Analog Out | This pin outputs 0-5V analog voltage based on the intensity of the gas. |
| For Sensor |  |  |
| 1 | H -Pins | Out of the two H pins, one pin is connected to supply and the other to ground |
| 2 | A-Pins | The A pins and B pins are interchangeable. These pins will be tied to the Supply voltage. |
| 3 | B-Pins | A pins and B pins are interchangeable. One pin will act as output while the other will be pulled to ground. |

### MQ-135 Sensor Features

- Wide detecting scope

- Fast response and High sensitivity

- Stable and long life

- Operating Voltage is +5V

- Detect/Measure NH3, NOx, alcohol, Benzene, smoke, CO2, etc.

- Analog output voltage: 0V to 5V

- Digital output voltage: 0V or 5V (TTL Logic)

- Preheat duration 20 seconds

- Can be used as a Digital or analog sensor

- The Sensitivity of Digital pin can be varied using the potentiometer

0.96 inch I2C/IIC 128x64 OLED Display Module 4 Pin

GND should be connected to the ground of Arduino

VCC is the power supply for the display which we connect the 5 volts pin on the Arduino.

SCL is a serial clock pin for I2C interface.

SDA is a serial data pin for I2C interface.

Connections:

- VCC pin to the 5V output on the Arduino

- GND to ground

- SCL is connected to pin A5 of arduino nano

- SDA is connected to pin A4 of arduino nano

TINERCAD PLAYGROUND :

Visit TINKERCAD to check live stimulation on tinkercad playground.

Also visit VIDEO for explanation on working for the same .

We have used default components which are present in tinkercad playground.( eg arduino uno)

Snapshots:

A smoke is used as demonstration to CO2 concentration .

When concentration is high near the sensor, display shows “HIGH PPM CONCENTRATION” and RED LED is turned ON .

When concentration is low near the sensor, display shows “MODERATE PPM CONCENTRATION” and GREEN LED is turned ON .

IMPLEMENTING THE PROJECT :

Please check out YOUTUBE (link) video for live demonstration.

CODE:

#include <SPI.h>

#include <MQ135.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

Adafruit_SSD1306 display(-1);

MQ135 gasSensor = MQ135(A0);

int val;

int sensorPin = A0;

int sensorValue = 0;

void setup() {

// initialize with the I2C addr 0x3C

gasSensor = MQ135(A0);

Serial.begin(9600);

pinMode(sensorPin, INPUT);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

}

void loop() {

// put your main code here, to run repeatedly:

display.setTextSize(3);

float rzero = gasSensor.getRZero();

float ppm = gasSensor.getPPM();

Serial.print ("ppm: ");

Serial.println (ppm);

display.setTextSize(1.2);

display.setTextColor(WHITE);

display.setCursor(0,28);

display.print("ppm   ");

display.println(ppm);

display.display();

delay(200);

display.clearDisplay();

}

SNAPSHOTS:

SERIAL OUTPUT  OF PPM CONCENTRATION :

GRAPHICAL REPRESENTION / ANALYSIS OF PPM CONCENTRATION

SETUP IN WORKING CONDITION :

LIMITATIONS:

  - The sensor (like commercial ones) undergoes some delay in measuring the concentration of CO2.

  - The measurement range is 0–9999 ppm. Concentrations not included in this range will not be measured

  - Extreme relative humidity, temperature, and voltage can affect the measurements

Checkout  Github ( link ) for the complete project .